Peripheral Interface for STM32F407G-DISC01

| Pin | Direction | Pin Function | Usage |
| --- | --- | --- | --- |
| PA0 | Input | GPIO | Onboard User Switch |
| PA1 | Output | Timer 2, Channel 2 | PWM |
| PA0 | Alternate Function AF8 | Tx | UART -> Bluetooth |
| PA1 | Alternate Function AF8 | Rx | UART -> Bluetooth |
| PA2 | Alternate Function AF7 | Tx | UART -> TTL |
| PA3 | Alternate Function AF7 | Rx | UART -> TTL |
| PD12 | Output | GPIO | Onboard Green LED |
| PD13 | Output | GPIO | Onboard Orange LED |
| PD14 | Output | GPIO | Onboard Red LED |
| PD15 | Output | GPIO | Onboard Blue LED |
| PB0 | Output | GPIO | Buzzer |
| PE1 | Output | GPIO | Stepper Motor |
| PE2 | Output | GPIO | Stepper Motor |
| PE3 | Output | GPIO | Stepper Motor |
| PE4 | Output | GPIO | Stepper Motor |
| PC1 | Input | Digital | DO of LDR |
| PC2 | Input | Analog | AO of LDR |